AI-Agent–Based Automation and Verification of Structural Analysis Workflows in Code_Aster

| Chapter | Pages | Pages reality! |
| --- | --- | --- |
| Intro | 7 |  |
| Lit Review | 18 | 12 |
| Methodology | 11 |  |
| Implementation | 15 |  |
| Case Study: Beam Analysis | 11 |  |
| Results and Discussion | 14 |  |
| Limitations and Challenges | 7 |  |
| Future Work |  |  |
| Conclusion |  |  |
| Appendices |  |  |
| Total | ~93–110 pages |  |

POLITECNICO DI TORINO

Corso di Laurea Magistrale

in Digital Skills for Sustainable Societal Transitions

Tesi di Laurea Magistrale

AI-Agent–Based Automation and Verification of Structural Analysis Workflows in Code_Aster

| Relatori |  | Candidato |
| --- | --- | --- |
| prof. Rosario Ceravolo ........................... prof. Gianvito Urgese ........................... prof. Gaetano Miraglia ........................... |  | Saeideh Mohammadikish ........................... |

A.A. 2024/2025

Table of Contents

Chapter 1 – Introduction	5

1.1 Background	5

1.2 Problem Statement	5

1.3 Objectives	5

1.4 Scope of the Work	5

1.5 Thesis Structure	5

Chapter 2 – Literature Review	6

2.1 FEM Workflows and Challenges	6

2.2 Visualization in Structural Analysis	7

2.3 Decision Support Systems	9

2.4 Human-in-the-Loop AI	10

2.5 LLMs in Engineering Applications	12

2.6 Research Gaps and Motivation	14

2.7 Summary of Findings	16

Chapter 3 – System Architecture & Pipeline	18

3.1 Overview of the Proposed System	18

3.2  System Workflow	18

3.3 System Components	18

3.4 Integration with FEM Outputs	18

Chapter 4 – Human-in-the-Loop Decision Support System	19

4.1 Overview of the Proposed System	19

4.2 Interaction Design	19

4.3 Input Handling & Validation	19

4.4 AI Interpretation and Decision Support	19

4.5 Hybrid Approach (Rule-based + LLM)	19

4.6 Explanation Generation	19

4.7 Human–AI Interaction Loop	19

4.8 System Workflow Example	19

Chapter 5 – Visualization Framework	20

5.1 Role in Decision Support	20

5.2 Implemented Visualization Tools	20

5.3 Result Types (Displacement, Stress, Forces)	20

5.4 Interactive Features (Zoom, Node Selection, Comparison)	20

Chapter 6 – Case Studies & Evaluation	21

6.1 Case Study Description	21

6.2 Experimental Setup	21

6.3 Scenario Definitions	21

6.4 Results	21

6.5 Analysis of Results	21

6.6 Discussion	21

6.7 Validation with Analytical Solutions	21

Chapter 7 – Discussion & Conclusion	22

7.1 Contributions of the Thesis	22

7.2 Limitations	22

7.3 Future Work	22

7.4 Final Remarks	22

References	23

# Chapter 1 – Introduction

## 1.1 Background

## 1.2 Problem Statement

## 1.3 Objectives

## 1.4 Scope of the Work

## 1.5 Thesis Structure

# Chapter 2 – Literature Review

# Chapter 3 –  Methodology

## 3.1 Overall System Architecture

The overall architecture of the proposed system is designed to simplify the structural analysis workflow in Code_Aster. In a typical setup, preparing a simulation involves several separate and often repetitive steps, such as defining the problem, creating input files, running the analysis, and checking the results. When these steps are performed manually, they can be time-consuming and prone to errors. For this reason, the system aims to organize these steps into a structured and repeatable workflow.

The proposed system consists of several main components, each with a specific role. First, the user provides the input parameters of the structural problem, such as geometry, boundary conditions, and loading. Then, an AI-based layer (LLM) processes this input and converts it into a structured format that can be used in the following steps. After that, Python-based modules take over. These modules are responsible for generating the required analysis files, including geometry, mesh, and especially the .comm file. They also handle the execution of the simulation in Code_Aster and manage the overall workflow.

Once the simulation is completed, the results are extracted. These results may include displacements, stresses, or other relevant quantities. A separate verification module is then used to compare the numerical results with reference values, such as analytical solutions or hand calculations. It is important to note that this step is intended only for validating the results, and not for supporting engineering decision-making.

The overall workflow follows a clear sequence. The process starts with user input, followed by data structuring and file generation. Then, the simulation is executed in Code_Aster, and the results are extracted. Finally, the results are compared with reference values. In this way, the system provides a complete pipeline from problem definition to result evaluation.

Finally, the system is designed in a modular way. Each component can be analyzed or improved independently. This makes the system easier to understand and also more flexible for future extensions.

### 3.1.1 Workflow overview

The workflow of the proposed system consists of several main stages that are executed sequentially and depend on each other. The purpose of defining this workflow is to establish a clear path from input definition to result evaluation, ensuring that the different steps of the analysis are performed in a structured and repeatable way.

In the first stage, the user provides the parameters of the structural problem. This input includes information such as geometry, boundary conditions, and loading. It is assumed that the input data is complete and ready to be used by the system. The input is then processed by the LLM-based layer, which organizes it into a structured format suitable for the following steps.

After structuring the input, the system proceeds to the file generation stage. In this step, Python modules are used to generate the necessary files required to run the analysis. These files include geometry definitions, mesh-related data, and especially the .comm file, which plays a central role in the execution process. The file generation process is designed to be structured and controlled.

In the next stage, the generated files are passed to Code_Aster. At this point, Code_Aster is executed as the solver, and the numerical analysis is performed based on the provided input files. The system itself does not perform the numerical computation but manages the execution of the analysis.

Once the execution is completed, the results are extracted. These results may include quantities such as displacements and stresses, which are used for further evaluation. A verification step is then carried out, where the numerical results are compared with reference values, such as analytical solutions or hand calculations. This step is intended to assess the accuracy of the results and does not involve engineering decision-making.

Overall, these stages form a continuous pipeline in which the output of each stage serves as the input for the next one. This structure makes the workflow repeatable, controlled, and suitable for further development.

### 3.1.2 Pipeline structure (Input → Execution → Verification)

The system can be represented as a three-stage pipeline consisting of Input, Execution, and Verification. This abstraction provides a simplified view of the system and helps clarify the role of each stage.

The Input stage involves receiving the problem parameters from the user and transforming them into a structured format. This stage serves as the starting point of the pipeline and prepares the data for further processing.

The Execution stage includes the preparation and running of the analysis. In this stage, the required files are generated, and Code_Aster is executed. This part represents the core operational layer of the system.

Finally, the Verification stage focuses on evaluating the results. The outputs are compared with reference values in order to assess their accuracy. This stage is used only for validation purposes.

This three-stage structure provides a high-level representation of the system, where each stage has a clear role and the flow between them is linear and well-defined.

### 3.1.3 Role of each component (LLM, Python, Code_Aster)

This section describes the role of each main component of the system. The goal is to clarify the responsibility of each part without repeating the overall workflow.

In this system, the LLM is used as a support layer for processing input data. Its main role is to transform the initial user input into a structured format that can be used in later stages. It does not perform numerical computations and is not involved in the execution of the analysis.

The Python modules form the core of the system. They are responsible for generating the required input files, managing the execution of Code_Aster, and extracting the results. In addition, the verification-related calculations are also implemented in this layer. This part can be considered the deterministic backbone of the system.

Finally, Code_Aster is used as the solver. It performs the numerical analysis based on the provided input files. The proposed system does not modify or replace the solver but instead focuses on preparing and executing the analysis.

This separation of responsibilities allows each component to operate within a clearly defined role. It also makes the system easier to maintain and extend.

## 3.2 Design Principles of the Proposed System

Several design principles were considered in the development of the proposed system to ensure that it remains reliable, controlled, and reasonably extensible. These principles influence how the different components of the system interact and operate.

One of the main principles is the separation of responsibilities between system components. In this structure, the LLM is used for input processing and structuring, while the execution and numerical computations are handled by Python modules and Code_Aster. This separation helps reduce dependencies and keeps each component focused on a specific task.

Another key principle is maintaining a deterministic core. The main operations, such as file generation, execution, and verification, are implemented in a predictable way and do not depend on the variable behavior of the LLM. This is important for ensuring consistent and repeatable results.

A further principle is the use of structured file generation methods. In particular, predefined templates are used for generating the .comm file, which helps reduce errors and makes the process more controlled.

Finally, the system is designed in a modular way, allowing each component to be analyzed or modified independently. This makes the system easier to maintain and extend.

## 3.3 User Input Processing

The user input processing stage refers to the step in which the initial problem data is prepared for use in the workflow. Since analysis in Code_Aster requires clear and structured inputs, the information provided by the user must be organized before it can be passed to the following stages.

In this study, the user input includes the main parameters of the structural problem, such as geometry, boundary conditions, and loading. These data may be provided in the form of plain text or short descriptive statements. For this reason, the input needs to be converted into a set of clearly defined engineering parameters before the analysis files are generated.

The purpose of this stage is not to change the content of the problem, but only to prepare it for later use. Therefore, input processing is considered a preliminary step between problem definition and analysis execution.

### 3.3.1 Natural language input

## 3.4 File Generation Module

## 3.5 Workflow Execution

## 3.6 Result Extraction

## 3.7 Verification Module

## 3.8 AI-Agent Design

# Chapter 4 – Human-in-the-Loop Decision Support System

## 4.1 Overview of the Proposed System

## 4.2 Interaction Design

## 4.3 Input Handling & Validation

## 4.4 AI Interpretation and Decision Support

## 4.5 Hybrid Approach (Rule-based + LLM)

## 4.6 Explanation Generation

## 4.7 Human–AI Interaction Loop

## 4.8 System Workflow Example

# Chapter 5 – Visualization Framework

## 5.1 Role in Decision Support

## 5.2 Implemented Visualization Tools

## 5.3 Result Types (Displacement, Stress, Forces)

## 5.4 Interactive Features (Zoom, Node Selection, Comparison)

# Chapter 6 – Case Studies & Evaluation

## 6.1 Case Study Description

## 6.2 Experimental Setup

## 6.3 Scenario Definitions

## 6.4 Results

## 6.5 Analysis of Results

## 6.6 Discussion

## 6.7 Validation with Analytical Solutions

# Chapter 7 – Discussion & Conclusion

## 7.1 Contributions of the Thesis

## 7.2 Limitations

## 7.3 Future Work

## 7.4 Final Remarks

# References

[1]	S. Youwai, D. Phim, V. G. Murcia, and R. C. Onas, “Large language model-based multi-agent systems for automated foundation design: router-driven task classification and expert selection framework,” AI Civ. Eng., vol. 5, no. 1, p. 5, Mar. 2026, doi: 10.1007/s43503-026-00088-8.

[2]	H. Xie, Q. Mei, and Y. H. Chui, “AI applications for structural design automation,” Autom. Constr., vol. 179, p. 106496, Nov. 2025, doi: 10.1016/j.autcon.2025.106496.

[3]	J. M. Huang, S. K. Ong, and A. Y. C. Nee, “Visualization and interaction of finite element analysis in augmented reality,” Comput.-Aided Des., vol. 84, pp. 1–14, Mar. 2017, doi: 10.1016/j.cad.2016.10.004.

[4]	M. Yavuz Erkek, S. Erkek, E. Jamei, M. Seyedmahmoudian, A. Stojcevski, and B. Horan, “Augmented Reality Visualization of Modal Analysis Using the Finite Element Method,” Appl. Sci., vol. 11, no. 3, p. 1310, Jan. 2021, doi: 10.3390/app11031310.

[5]	N. Catbas, S. Ciloglu, O. Hasançebi, K. Grimmelsman, and A. Aktan, “Limitations in Structural Identification of Large Constructed Structures,” J. Struct. Eng., vol. 133, Aug. 2007, doi: 10.1061/(ASCE)0733-9445(2007)133:8(1051).

[6]	A. Felsberger, B. Oberegger, and G. Reiner, “A Review of Decision Support Systems for Manufacturing Systems”.

[7]	B. W. Morrison et al., “Decision Support Systems (DSSs) ‘In the Wild’: The Factors That Influence Users’ Acceptance of DSSs in Naturalistic Settings,” J. Cogn. Eng. Decis. Mak., vol. 17, no. 4, pp. 332–350, Dec. 2023, doi: 10.1177/15553434231191385.

[8]	M. J. Miller, K. M. McGuire, and K. M. Feigh, “Decision Support System Requirements Definition for Human Extravehicular Activity Based on Cognitive Work Analysis,” J. Cogn. Eng. Decis. Mak., vol. 11, no. 2, pp. 136–165, Jun. 2017, doi: 10.1177/1555343416672112.

[9]	E. Papagiannidis, P. Mikalef, and K. Conboy, “Responsible artificial intelligence governance: A review and research framework,” J. Strateg. Inf. Syst., vol. 34, no. 2, p. 101885, Jun. 2025, doi: 10.1016/j.jsis.2024.101885.

[10]	T. Herrmann and S. Pfeiffer, “Keeping the organization in the loop: a socio-technical extension of human-centered artificial intelligence,” AI Soc., vol. 38, no. 4, pp. 1523–1542, Aug. 2023, doi: 10.1007/s00146-022-01391-5.

[11]	J. Jiang, A. J. Karran, C. K. Coursaris, P.-M. Léger, and J. Beringer, “A Situation Awareness Perspective on Human-AI Interaction: Tensions and Opportunities,” Int. J. Human–Computer Interact., vol. 39, no. 9, pp. 1789–1806, May 2023, doi: 10.1080/10447318.2022.2093863.

[12]	G. Culot, M. Podrecca, and G. Nassimbeni, “Artificial intelligence in supply chain management: A systematic literature review of empirical studies and research directions,” Comput. Ind., vol. 162, p. 104132, Nov. 2024, doi: 10.1016/j.compind.2024.104132.

[13]	J. Chen and Y. Bao, “Multi-agent large language model framework for code-compliant automated design of reinforced concrete structures,” Autom. Constr., vol. 177, p. 106331, Sep. 2025, doi: 10.1016/j.autcon.2025.106331.

[14]	J. R. Schoenherr and K. Michael, “Multi-Level Sociotechnical Systems Approach to Human-Centered AI,” in Handbook of Human-Centered Artificial Intelligence, Springer, Singapore, 2026, pp. 1–74. doi: 10.1007/978-981-97-8440-0_100-1.

[15]	A. Chakir, J. F. Andry, A. Ullah, R. Bansal, and M. Ghazouani, Eds., Engineering Applications of Artificial Intelligence. in Synthesis Lectures on Engineering, Science, and Technology. Cham: Springer Nature Switzerland, 2024. doi: 10.1007/978-3-031-50300-9.